## VLAN Distribution (Keep it Logical)

| Access Switch | VLANs | Redundancy |
| --- | --- | --- |
| SW2 | VLAN 10 (Eng), VLAN 20 (Sales) | Dual-homed to SW1/SW5 |
| SW3 | VLAN 10 (Eng), VLAN 30 (Admin) | Dual-homed to SW1/SW5 |
| SW4 | VLAN 20 (Sales), VLAN 30 (Admin) | Dual-homed to SW1/SW5 |
| SW6 | VLAN 10 (Eng), VLAN 20 (Sales), VLAN 30 (Admin) | Dual-homed to SW1/SW5 |

## Recommended Lab Progression

### Week 1: Physical Setup

- □ Connect R1-R2 link

- □ Connect SW1-SW5 link

- □ Connect SW2/SW3/SW4/SW6 to both SW1 & SW5

### Week 2: Layer 2

- □ VLANs on all switches

- □ Trunk configuration (native VLAN 199)

- □ EtherChannel on access switches

- □ STP verification (find root bridge)

### Week 3: Layer 3

- □ SVIs on distribution switches

- □ HSRP for all VLANs

- □ OSPF between routers and distribution

Week 4: Testing & Troubleshooting

- □ Failover tests (pull cables!)

- □ Verify traffic paths

- □ Tune timers for faster convergence

- □ Document everything

EXCELLENT plan! 🎯 This is exactly how professionals learn - iterative design starting with L2 distribution (simpler) then evolving to L3 distribution (more advanced). You'll truly understand why L3 distribution is better by experiencing L2 limitations first!

## Your Learning Path - Master Plan

text

Week 1-2: L2 Distribution (2960s)

↓

Week 3-4: L3 Distribution (3650s)

↓

Week 5: Comparison & Documentation

Why this works: You'll appreciate L3 distribution SO much more after dealing with STP convergence and router-on-a-stick bottlenecks! 😄

# PHASE 1: L2 DISTRIBUTION COMPLETE CONFIGS

## Network Overview

text

ISP1                    ISP2

|                       |

[R1]===================[R2]    ← HSRP on routers

/|\                    /|\

/ | \                  / | \

/  |  \                /  |  \

[SW1]====[SW5]          ← L2 Distribution (2960s)

/|\    /|\

/ | \  / | \

/  |  \/  |  \

[SW2][SW3][SW4][SW6]          ← Access Switches (Dual-homed)

|    |    |    |

PCs  PCs  PCs  PCs

## IP Addressing Plan

### WAN Links

| Link | Network | R1 IP | R2 IP | ISP IP |
| --- | --- | --- | --- | --- |
| R1-ISP1 | 203.0.113.0/30 | .2 | - | .1 |
| R2-ISP2 | 203.0.113.4/30 | - | .6 | .5 |
| R1-R2 | 10.0.0.0/30 | .1 | .2 | - |

### Inter-Switch Links (Trunks)

| Link | VLANs Allowed |
| --- | --- |
| SW1-SW5 | 10,20,30,99,199 |
| SW2-SW1/SW5 | 10,20,99,199 |
| SW3-SW1/SW5 | 10,30,99,199 |
| SW4-SW1/SW5 | 20,30,99,199 |
| SW6-SW1/SW5 | 10,20,30,99,199 |

### VLANs & Subnets

| VLAN | Name | Network | Gateway (HSRP) |
| --- | --- | --- | --- |
| 10 | Eng | 192.168.10.0/24 | 192.168.10.1 |
| 20 | Sales | 192.168.20.0/24 | 192.168.20.1 |
| 30 | Admin | 192.168.30.0/24 | 192.168.30.1 |
| 99 | Management | 192.168.99.0/24 | 192.168.99.1 |
| 199 | Native | Unused | - |

### IPv6 Addressing

| VLAN | Network | Gateway |
| --- | --- | --- |
| 10 | 2001:db8:0:10::/64 | ::1 |
| 20 | 2001:db8:0:20::/64 | ::1 |
| 30 | 2001:db8:0:30::/64 | ::1 |
| 99 | 2001:db8:0:99::/64 | ::1 |
| R1-R2 | 2001:db8:0:1::/64 | - |
| R1-ISP1 | 2001:db8:0:113::/64 | - |
| R2-ISP2 | 2001:db8:0:114::/64 | - |

Link Local Address Reference::: Begin

! ISP

interface g0/0/0: fe80::101    ! ISP-R1 link

interface g0/0/1: fe80::102    ! ISP-R2 link

! R1

interface g0/0/0: fe80::1      ! R1-ISP link

interface g0/0/1: fe80::2      ! R1-R2 link

interface g0/0/2: fe80::3      ! R1-SW1 link

interface g0/0/3: fe80::4      ! R1-SW5 link

! R2

interface g0/0/0: fe80::2      ! R2-ISP link

interface g0/0/1: fe80::3      ! R2-R1 link

...

Link Local Address Reference::: End

All L2 Switch 2960-24TT w/o channel-group capability.

2960s in PT DO NOT support channel-group. For EtherChannel in Packet Tracer, use 3650 switches instead!

### Later for channel-group capability Use 3650s for Access Switches too

If you want EtherChannel, replace all 2960s with 3650s in PT. They'll still work as L2 switches with no ip routing.